General goal: get familiar with the code: where does what?

Problem 5

In this tutorial, you will implement the PDE of mixture fraction (f) in the simple chemical reacting system (SCRS) model.

∂(ρf)∂t+divρfu=div(Γfgradf)

Tips 1. List the new variables and constants

Tips 2. What are the initial and boundary conditions for f (implement in bound.m)?

Tips 3. Make a new function file (fcoeff.m)

Simulate a laminar non-premixed diffusion flame as in the geometry shown below. Let us consider burning of a pure methane jet with co-flowing air.

CH4+2(O2+3.76N2) → CO2+2H20+(2×3.76)N2

L=10 cm

D=2.5 cm

Pure methane inlet

d=0.5 cm, u=0.2 m/s

Air inlet

u=0.2 m/s